MIDIEX.EXE                28-MAR-86 40675(17376)       ACCESSES: 19

MIDIEX IS A PATCH EXCHANGE UTILITY FOR THE IBM-PC EQUIPPED WITH AN

MPU-401 INTERFACE.  IT SENDS AND RECEIVES MIDI SYSTEM EXCLUSIVE

DUMPS IN A PLAIN FORMAT: HEX F0, MANUFACTURER'S ID, DATA, AND EOX

(HEX F7).  YOU CAN USE THIS UTILITY TO CAPTURE A DX7 PATCH DUMP,

FOR EXAMPLE, AND UPLOAD IT TO THIS DL FOR US ALL TO SHARE.

LIKEWISE IT WILL TRANSFER DOWNLOADED PATCHES TO YOUR SYNTHESIZER.

** UPDATED 3/28/86 TO WORK WITH RX DRUM MACHINES, AND OTHER SYNTHS.

** FILE IS 16K.

=========================================================================

Additional notes by Chris Bosshardt

Capitol MIDI Users' Group

301-984-0024 (v)

301-881-6323 (d)

Note: To use this program, first turn on your computer and your DX7 or

TX7, and make sure they are cabled together properly.  Next,

follow these steps:

Press the DX7's FUNCTION key

Press voice-select key #8 (it says "Feedback" above it)

You should see the phrase "MIDI CH=1" in your LCD display.

Press voice-select key #8 a second time

You should see the phrase "SYS INFO UNAVAIL" in your LCD display.

Press the YES switch

You should see the phrase "SYS INFO AVAIL" in your LCD display.

Press the MEMORY PROTECT/INTERNAL switch

You should see the phrase "INTERNAL ON" in your LCD display.

Press the NO switch

You should see the phrase "INTERNAL OFF" in your LCD display.

You're now ready to load and use MIDIEX.  When you use it, be

sure include the file extension of .DX7 when specifying the name

of the voice file to be loaded or saved.

========================================================================