------------------>>>>>>>Getit!<<<<<<<<<<------------------------------

-----------------Generic Midi batch file librarian---------------------

Copyright (c) 1987 Aron Nelson

1800 Stokes St. #197

San Jose, Ca 95126

Getit! is a generic batch file compatible librarian that can negotiate

a wide variety of midi system exclusive dumps. To run Getit! you must

know a few things about your MIDI instrument. You must know if your

synthesizer can be prompted to send a dump (an old dx7 cannot) and

if it can, you must know what header bytes to send to negotiate the dump.

You must know the size of the dunp and the first byte that is dumped from

the synth. These items can usually be found in the instruments owners

manual under <system exclusive data format>.Once these items are known

then execute Getit! by creating a batch file i.e.:

A> copy con example.bat

Getit <first byte back from synth> <size of dump> <header1>..<header2>..<etc>..

<press F6 when finished.>

Getit! will send out the header bytes <header1> <header2> etc. then assuming

then headers are correct, the synth will send a dump back to the computer.

Getit!  will filter out all bytes received until it gets the first byte

back from the synth that you assigned. This is necessary because some synths

send out I.D. bytes periodically.(you don't want those bytes!)

Getit will store all bytes until a decimal 247 which is defined in the MIDI

spec as the end of a system exclusive message. If the dump matches the size

that you specified then Getit will ask you if you want to save the data.

If you answer yes Getit! will then ask you if you want an offset value. This

offset is used if you want to truncate some header bytes off of the beginning

of the system exclusive dump. Some synths send a header that you might not

want, usually you would answer this  as no. Getit! will then ask you if

you want to strip off the checksum and end of exclusive(247), again

generally you would answer no to this. Getit! will then save your file.

That's it!

************************************************************************

I hope that you find it usefull.

If you use and like Getit! and Sendit! a $10 donation is required.

************************************************************************

Notes:

If you create a batch file:

A> copy con Dummy.bat

Getit 240 500 (actually anything large)  0 0 0 (dummy bytes)

<F6 when finished>

then prompt your synth manually to dump its data,  you can find

how large the dump is. (of course this strips off any headers that would

be in the dump). (useful for dx7 type instruments which cannot be prompted

to dump its internal data)

--------------------------->>>>>Sendit!<<<<<--------------------------------------

To send a MIDI file back to Synth -> run sendit:

just type 'sendit' at the dos prompt.

Sendit! will ask you for the name of the file, just type in the name of the

file that you want to send.

************************* DISCLAIMER **************************************

I do not assume any liability for the use of Getit! and Sendtit! at all!.

In no event will I be liable to you for any damages including lost profits

that arise out of the use of Getit! and Sendit! even if I have been

advised of the possibility of such damages.

**************************************************************************

*****NOTE*******

The example .bat files are for the Yamaha tx81z tone generator.

they also work with the dx100 series too.