MPU.LIB  	for Quick Basic 4.00

Assembly Subroutines to communicate

with the MPU-401.

Michel Poirier (c) 1988

for MIDICOM BBS, Montreal, Quebec.

FidoNet ID: 167/111

(514) 744-7354

--------------------------------------------------------------------

Start Quick Basic 4.00 with option "/L" :   QB /L MPU.QLB

--------------------------------------------------------------------

SendCmd Cmd%

Sends a one byte Command to MPU-401.

Cmd% = Integer Variable

ReadData MidiIn%

Reads one byte from the MPU-401.

MidiIn% = Integer Variable

SendData LEN(MidiOut$), SADD(MidiOut$)

Sends a String of Data to MPU-401.

MidiOut$ = String of Data